Sarhad University of Science and Information Technology, Peshawar

Department of CS/IT

Final Term Lab Examination 2025

Microprocessor Architecture & Assembly Language

4th Semester

Total Marks: 30								      Instructor: Muhammad Danish Ali

Name: Ahmad								Registration Number: SU-23-01-001-005

Note: Attempt all questions, each question carries equal marks.

- Choose the Correct Answer									          [CLO-1]

- What does the JG instruction do?
A) Jump if less than
B) Jump if greater
C) Jump if equal
D) Jump always

Answer: A

- What is the purpose of the instruction mov bl, 3 in the program?
A) It stores the result of comparison
B) It clears the BL register
C) It loads the value 3 into BL for comparison
D) It adds 3 to AL

Answer: C

- What is the purpose of the CMP AL, BL instruction?
A) To add AL and BL
B) To compare AL with BL
C) To multiply AL and BL
D) To move BL into AL

Answer: B

- What is the value of AL immediately after mov al, 8 executes?
A) 0
B) 1
C) 8
D) 3

Answer: C

- What instruction stops the program execution?
A) MOV
B) CMP
C) JG
D) HLT

Answer: D

- Write an assembly program to multiply two signed 8-bit numbers: 73H and 10H, using registers AL and BL. Show the result in AX.										          [CLO-2]

Answer:
mov al, 73h

mov bl, 10h

cbw

imul bl

- Write an assembly program to divide an unsigned 16-bit number 0021H by 0005H. Use AX as the dividend and BX as the divisor.											          [CLO-2]

Answer:

mov ax, 0021h

mov bx, 0005h

xor dx, dx

div bx

- Write an assembly program to display the message "Microprocessor and Assembly Language" on the screen.

[CLO-2]

Answer:
mov ah, 09h

mov dx, offset msg

int 21h

int 20h

msg db 'Microprocessor and Assembly Language$', 0

- Write an assembly program that compares two values 5 and 6 in AL and BL registers. Use the JNE instruction to jump if they are not equal. Assign AL a value of 9 if not equal, otherwise 0.			          [CLO-2]

Answer:
mov al, 5

mov bl, 6

cmp al, bl

jne not_equal

mov al, 0

jmp done

not_equal:

mov al, 9

done:

- Write an assembly program that compares two signed numbers 8 and 3 in AL and BL registers. Use the JG instruction to jump if AL is greater than BL. Set AL to 1 if greater, otherwise set AL to 0. End the program properly. 												         [CLO-2]

Answer:
mov al, 8

mov bl, 3

cmp al, bl

jg greater

mov al, 0

jmp endp

greater:

mov al, 1

endp: